Release scope: global

Internal: Global Service Department

External: overseas authorized service centers (ASCs) and high-level repair centers (HLRCs)

Technical Announcement for

Columbia (Honor 10 Smart Phones)

To all ASCs :

This document aims to help ASCs identify the model of a phone or PCBA without disassembling the phone

Contents

1 Spare Parts Identification	3

1.1 Phone Identification	3

1.1.1 Identification by the Laser-Engraved Code on the Rear Cover	3

1.2 Identification by the Device Storage Space	4

2 PCBA Identification	5

2.1 Identification by Silkscreen	5

2.2 Identification by the Device Storage Space	5

2.3 Identification by Label Laser Engraving Code on the Board Barcode Label or the BOM Code	7

3 Subsidiary Board Identification	7

4 Maintenance Strategies	8

4.1 Maintenance Strategies for ASCs	8

5 FAQs	12

There are three internal models of Honor Columbia phones for regions outside China:

Columbia-L29A, Columbia-L29D, and Columbia-AL10I (India edition)

| Name | Luxury full-frequency India edition | General standard edition for regions outside China | General premium edition for regions outside China |
| --- | --- | --- | --- |
| External Model | COL-AL10 | COL-L29 | COL-L29 |
| Internal Model | Columbia-AL10I | Columbia-L29A | Columbia-L29D |
| Carrier | India | Regions outside China | Regions outside China |
| Single/Dual SIM Cards | Dual SIM cards | Dual SIM cards | Dual SIM cards |
| Brand (Rear Cover) | honor(No laser engraving ) | honor(With laser engraving ) | honor(With laser engraving ) |
| Configuration | 6 GB+128 GB | 4 GB+64 GB | 4 GB+128 GB |
| PCBA Version | HL1COLM | HL2COLM | HL2COLM |

Spare Parts Identification

Phone Identification

This announcement is prepared to guide all ASCs on how to identify the phone model.

Identification by the Laser-Engraved Code on the Rear Cover

Each handset's model code is laser-engraved on the glass rear cover, as shown in Figure 1-1.

Mapping between the laser-engraved code and phone model

India edition model is laser-engraved on the glass rear cover label, as shown in Figure 1-2.

Mapping between the laser-engraved code and phone model

The matching between the code and model is described in Table 1-1.

Mapping of model code and phone model

| Applicable Scope | External Model (Verified Model) | Internal Model | Edition |
| --- | --- | --- | --- |
| Outside China | COL-L29 | Columbia-L29A | Dual-SIM 4 GB+64 GB standard edition |
|  |  | Columbia-L29D | Dual-SIM 4 GB+128 GB advanced edition |
|  | COL-AL10 | Columbia-AL10I | Dual-SIM 6 GB+128 GB India privilege edition |

If the phone can power on properly, choose Settings > System > About phone to check the model code, RAM, and ROM to identify the phone model. If the phone cannot power on properly, disassemble the device, check the board barcode or DDR version to identify the device model and record the order.

Identification by the Device Storage Space

Check the device storage space to further identify the phone model. There are two types of storage space for Columbia phones: 64 GB and 128 GB.

If the phone can power on properly, choose Settings > Storage to check the storage space.

Refer to the external model printed on the rear cover, the device storage space, and Table 1-1 to identify the internal phone model.

PCBA Identification

Identification by Silkscreen

Each PCBA has its silkscreen, as shown in Figure 2-1. The matching between the silkscreen and model is described in Table 2-1. PCB is divided into PCB1 and PCB2.

PCBA silkscreen position

Matching of PCBA code, PCBA silkscreen, and spare board code

| Internal Model | PCBA Code | PCBA Silkscreen | Spare Board Code | Remarks |
| --- | --- | --- | --- | --- |
| Columbia-L29A | 03025DYS | HL2COLM | 03032WUY | PCB2 |
| Columbia-L29D | 03025DYT | HL2COLM | 03032WVA | PCB2 |
| Columbia-AL10I | 03025DYL | HL1COLM | 03032XGS | PCB1 |

Identification by the Device Storage Space

If the device cannot power on, check the silkscreen on the ROM storage chip of the PCBA to identify the storage space. The location of PCB board is shown in Figure 2-2 marked as U1400. The mapping is shown in Table 2-2. Refer to Table 2-1 and check the PCB silkscreen to identify the device edition.

ROM storage chip silkscreen

The mappings of the ROM storage chip silkscreen and storage space

| PCB | Storage Configuration | Designator | Manufacturer | Manufacturer Model Code |
| --- | --- | --- | --- | --- |
| HL1COLM | 64 GB | U1400 | TOSHIBA | THGAF4G9N4LBAIR |
|  |  |  | SAMSUNG | KLUCG4J1ED-B0C1003 |
|  | 128 GB |  | TOSHIBA | THGAF4T0N8LBAIR |
|  |  |  | SAMSUNG | KLUDG4U1EA |
| HL2COLM | 64 GB | U1400 | TOSHIBA | THGAF4G9N4LBAIR |
|  |  |  | SAMSUNG | KLUCG4J1ED-B0C1003 |
|  | 128 GB |  | TOSHIBA | THGAF4T0N8LBAIR |
|  |  |  | SAMSUNG | KLUDG4U1EA |

Identification by Label Laser Engraving Code on the Board Barcode Label or the BOM Code

There is a board barcode label on the PCBA of every Honor sub-model. The board SN and product model are printed on the board barcode label. Check the label for identification. Or, check the BOM code associated with the board SN on ComPartner, then enter the BOM code into the CCP system to identify the model.

Board barcode location

Subsidiary Board Identification

All international edition devices share the same type of PCB. For the edition in India, the C7041component on the subsidiary board is an empty solder pad. For other editions outside China, the C7041 component is a 100 kΩ resistor, as shown in the following figure.

Maintenance Strategies

Maintenance Strategies for ASCs

All the materials involving replacement in the following table have been granted ASC in–warranty authorities. Do not maintain unauthorized materials.

| No. | BOM Code | Faulty Component | Maintenance Solution |  | Remarks |
| --- | --- | --- | --- | --- | --- |
|  |  |  | In-Warranty Maintenance | Out-of-Warranty Maintenance |  |
| 1 | 23060293 | Front camera module | Replace the module. | Replace the module. | Applicable to all models |
| 2 | ∕ | TP/LCD | Replace the front cover assembly. | Replace the front cover assembly. | Apply for the new component based on the BOM code. |
| 3 | 22030072 | Receiver | Replace the receiver. | Replace the receiver. | Apply for foam when replacing the receiver (BOM number: 51636932) |
| 4 | 32050072 | Motor | Replace the motor. | Replace the motor. | Apply for the code based on the device BOM. |
| 5 | 51661HYT | Receiver decorating part | Replace the front cover assembly. | Replace the front cover assembly. | Apply for the code based on the device BOM. |
| 6 | 03024WCV | Side button FPC | Replace the side button FPC. | Replace the side button FPC. | Apply for the code based on the device BOM. |
| 7 | 03024XFX | Main FPC | Replace the main FPC. | Replace the main FPC. | The component can be reused if no deformation is found. |
| 8 | 24022573 | Battery | Replace the front cover assembly. | Replace the battery. | Apply for adhesive when replacing the battery (BOM number: 51638384) |
| 9 | ∕ | Rear cover assembly | Replace the rear cover assembly. | Replace the rear cover assembly. | Apply for the code based on the device BOM. |
| 10 | ∕ | Antenna sub-board assembly | Replace the antenna sub-board assembly. | Replace the antenna sub-board assembly. | Apply for the code based on the device BOM. |
| 11 | ∕ | Fingerprint sensor module | Replace the front cover assembly. | Replace the front cover assembly. | Apply for the code based on the device BOM. |
| 12 | 03024WDP | Proximity sensor | Replace the front cover assembly. | Replace the front cover assembly. | Apply for the code based on the device BOM. |
| 13 | ∕ | PCBA (service backup board) |  | Replace the PCBA. | Apply for the code based on the device BOM. |
| 14 | 14241345 | RF connector | Replace the RF connector. | Replace the RF connector. | Apply for the code based on the device BOM. |
| 15 | 22020308 | Speaker module | Replace the speaker module. | Replace the speaker module. | Replace the adhesive after disassembly. Apply for the code based on the device BOM. |
| 16 | 23060308 | Rear camera module | Replace the module. | Replace the module. | Apply for the code based on the device BOM. |
| 17 | ∕ | SIM tray | Replace the card tray. | Replace the card tray. | Apply for the code based on the device BOM. |
| 18 | 22050181 | Main microphone | Replace the USB sub-board. | Replace the USB sub-board. | Applicable to all versions. |
| 19 | 14241124 | SIM card holder | Replace the PCBA. | Replace the card holder. | Applicable to all versions. |
| 20 | 22050181 | Secondary microphone | Replace the PCBA. | Replace the secondary microphone. | Applicable to all versions. |
| 21 | 51661JJU | Side button support assembly | Replace the support assembly. | Replace the support assembly. | Applicable to all versions. |
| 22 | 51661JAA | USB rubber cover | Replace the rubber cover. | Replace the rubber cover. | Applicable to all versions. |
| 23 | 51661JAB | Headset rubber cover | Replace the rubber cover. | Replace the rubber cover. | Applicable to all versions. |
| 24 | / | Volume button | Replace the button. | Replace the button. | Apply for the code based on the device BOM. |
| 25 | / | Power button | Replace the button. | Replace the button. | Apply for the code based on the device BOM. |
| 26 | 51629428 | Main FPCBTB connector fixing metal plate assembly | Replace the metal plate assembly. | Replace the metal plate assembly. | Applicable to all versions. |
| 27 | 51638013 | Rear cover graphite sheet | Replace the graphite sheet. | Replace the graphite sheet. | Applicable to all versions. |
| 28 | 51661HYS | Support assembly | Replace the support assembly. | Replace the support assembly. | Applicable to all versions. |
| 29 | 51638387 | Four-in-one rear cover adhesive | Replace the adhesive. | Replace the adhesive. | Applicable to all versions. |
| 30 | 51638384 | Battery adhesive | Replace the battery adhesive. | Replace the battery adhesive. | Applicable to all versions. |
| 31 | 51638389 | The mark-resistant adhesive on the upper side of the rear cover | Replace the mark-resistant adhesive. | Replace the mark-resistant adhesive. | Applicable to all versions. |
| 32 | 51638390 | The mark-resistant adhesive on the bottom half of the battery cover | Replace the mark-resistant adhesive. | Replace the mark-resistant adhesive. | Applicable to all versions. |
| 33 | 51638394 | Speaker fixing double-sided tape | Replace the double-sided tape. | Replace the double-sided tape. | Applicable to all versions. |
| 34 | 03024WDF | Flash FPC | Replace the flash FPC. | Replace the flash FPC. | Applicable to all versions. |
| 35 | 51661JYV | MAIN-CAMERA-LENS | Replace the CAMERA-LENS | Replace the CAMERA-LENS | Applicable to all versions. |

Materials that Cannot Be Reused Once Removed

| No. | Material | Remarks |
| --- | --- | --- |
| 1 | Receiver | This material cannot be reused once removed. |
| 2 | Motor | This material cannot be reused once removed. |
| 3 | Battery | This material cannot be reused once removed. |
| 4 | Radio frequency connector | This material cannot be reused once removed. |
| 6 | Shielding cover | This material cannot be reused once removed. |
| 7 | Foam and adhesive | This material cannot be reused once removed. |
| 8 | Main FPC | This material can be reused if no deformation is found. |

FAQs

After the product is upgraded to some version , the historical minor version rollback is no longer supported issue .

Issue Description

After the product is upgraded to the version listed in Table 1, the historical minor version rollback is no longer supported.

| Table 1 List of products and corresponding versions |  |
| --- | --- |
| Product | Upgrade-software Version |
| P20 | If the product in the general region is upgraded to B120 or a later version, the historical version cannot be rolled back. The areas that need to be specially noted are as follows: France -Orange, Spain -Orange products cannot be rolled back to the historical version after upgrading to B129 and later Mexico ATT, Mexico TELCEL products cannot be rolled back to the historical version after upgrading to B103 and later New Zealand products cannot be rolled back to the historical version after upgrading to B128 and later Canada products cannot be rolled back to the historical version after upgrading to B104 and later Taiwan products cannot be rolled back to the historical version after upgrading to B128 and later |
| P20Pro | If the product in the general region is upgraded to B120 or a later version, the historical version cannot be rolled back. The areas that need to be specially noted are as follows: UK channel, UK EE, Italy wind, Italy H3G, UK H3G products cannot be rolled back to the historical version after upgrading to B110 and later Mexico TELCEL products cannot be rolled back to the historical version after upgrading to B103 and later Canada products cannot be rolled back to the historical version after upgrading to B109 and later |
| MateRS | The products cannot be rolled back to the historical version after upgrading to B120 and later |
| Honor10 | The products cannot be rolled back to the historical version after upgrading to B120 and later |

Take P20 as an example. In addition to the exception areas listed in Table 1, after the version is upgraded to B120 or later, the earlier versions cannot be rolled back. That is, the earlier versions earlier than B120 cannot be rolled back.

Issue Solution

Upgrading the P20, P20Pro, MateRS, Honor10 should use the versions listed in Table 1.

Reply message: Rolling back the earlier version may cause unstable running of the mobile phone. You are advised to use the latest version.

Global Services Department

Sep.7.2018